SEQUENCE OF TROUBLE-SHOOTING FOR BRAKE PEDAL HARD / VACUUM NOT HOLDING

# TMC MOUNTING  ON BOOSTER SHOULD BE SECURLY  TIGHT

VACCUM HOSE SHOULD BE SECURLY TIGHT WITH CLAMP AT BOOSTER SIDE.

VACCUM HOSE SHOULD BE SECURLY TIGHT WITH CLAMP AT VACCUM PUMP SIDE.

VACCUM HOSE SHOULD NOT BE KINKED.

VACCUM PUMP ELBOW BOLT SECURLY TIGHT.

IF ALL ABOVE PARAMETERS ARE OK, AND STILL BRAKE PEDAL HARD / VACCUM NOT HOLDING, REPLACE BRAKE BOOSTER.