To All Service Engineers   & Regional Managers      26/09/03

SUB :- MODIFICATION OF NCR TURBO ENGINE VACCUM

SHUT OFF VALVE MOUNTING BRACKET.

We are enclosing the sketch Showing Modified Vacuum shut off valve mounting bracket, implemented on Trax NCR Turbo Engines. The code nos. of modified parts are also enclosed for your ready reference.

In case of incorrect operation of Vacuum shut off mechanism , e.g .engine shutting off but poor restarting reported you are advised to replace with  the modified bracket. This has been already implemented on Production line with effect From Chassis no T55020790 J02

The Part Nos. are

1) VACCUM SHUT OFF VALE -          B00 808 070 00 46 THIS IS INTERCHANGABLE

- CRANK LEVER ASSEMBLY-            B00 808 070 11 21

3) JOINT ROD ASSEMBLY -               B00 808 070 12 75

4) SUPPORT BRACKET    -                 B00 808 076 00 15

5) BRACKET FOR SHUT OFF VALVE -  B00 808 076 05 40

PLEASE CONVEY THIS  INFORMATION  TO ALL CONCERNED

IN YOUR TRRITORY

S V SHRIHITTIKAR                                         V N BELLARY

DIVISIONAL  MANAGER  (SERVICE)             DY MANAGER

(SERVICE)